Title: Speed-Based Detection of Objects Moving Away Using a Horizontally Oriented Ultrasonic Sensor

Project Type: Project in Machine Learning

## Assigned Student:  Attar, Saquib saquib.attar@stud.fra-uas.de, Hanumanthu, Manoj manoj.hanumanthu@stud.fra-uas.de, Ijaz, Muhammad Ahsan muhammad.ijaz@stud.fra-uas.de, Patel, Aman Basha aman.patel@stud.fra-uas.de

## Research Problem / Hypothesis / Motivation

Ultrasonic sensors are widely used in robotics, industrial automation, and intelligent systems for short-range distance measurement due to their low cost, robustness, and independence from lighting conditions. In many real-world applications, such as conveyor belt monitoring, warehouse robotics, and safety systems, it is not sufficient to detect only the presence of an object. Instead, systems must be capable of identifying movement direction and estimating the speed at which objects move.

While object approach detection is commonly addressed in ultrasonic-based systems, the reliable detection of objects moving away (AWY) and accurate speed estimation remains challenging. Ultrasonic measurements are affected by noise, reflections, sensor angle, and environmental conditions, which can degrade the accuracy of speed estimation derived from time-of-flight (TOF) data.

Previous research has shown that object velocity can be estimated using ultrasonic distance measurements by analyzing distance changes over time and applying regression-based methods [1], [2]. However, many existing approaches focus on complex sensor setups or multi-sensor configurations. There is a need for a simple, explainable, and experimentally validated approach that uses a single horizontally oriented ultrasonic sensor to detect AWY movement and estimate object speed reliably.

This project aims to address this gap by developing and evaluating a speed-based AWY detection system using ultrasonic TOF measurements, focusing on robustness, simplicity, and practical applicability.

## Goal / Objective

The primary goal of this project is to design and evaluate a system that can:

- Detect objects moving away (AWY) from a horizontally oriented ultrasonic sensor

- Estimate the speed of the moving object using distance-time measurements

- Reduce the influence of noise through signal preprocessing

- Evaluate the accuracy and reliability of the proposed approach using quantitative metrics

Successful completion of the project means:

- AWY movement is detected reliably

- Object speed is estimated with acceptable error

- The method works consistently across different speeds and distances

- Results are clearly validated through experiments and evaluation metrics

## Methodology

The project will begin with a structured literature review on ultrasonic-based motion detection and speed estimation techniques, focusing on TOF-based methods and regression approaches [1]-[3]. This will establish the state of the art and justify the chosen methodology.

The experimental setup will use a smart ultrasonic sensor (FIUS) mounted horizontally. Objects will be moved away from the sensor at controlled speeds and distances. The sensor will record TOF measurements, which are converted into distance values.

The methodology consists of four main stages:

- Data Acquisition
Distance measurements will be sampled at a fixed rate while objects move away from the sensor. Multiple trials will be recorded for different speeds and distances to ensure statistical validity.

- Signal Processing
Raw distance data will be processed using filtering techniques such as moving average or median filtering to reduce noise and outliers.
In addition, a time-window-based echo detection step is applied to distinguish valid ultrasonic echoes from noise. Distance samples are analyzed within a sliding window, and only stable windows classified as valid echo frames are retained. The center of the detected echo frame is selected as the reference point for further processing.

- Speed Estimation and AWY Detection
Speed estimation will be performed using linear regression, where the slope of the distance-time curve represents object speed:

d(t)=d0+v⋅t

The regression process is applied only after a valid echo frame has been detected, using distance measurements starting from the center of the detected frame. AWY detection is achieved by analyzing the sign and consistency of the distance gradient.

- Evaluation
Performance will be evaluated using metrics such as Mean Absolute Error (MAE), Root Mean Square Error (RMSE), and AWY detection accuracy. The influence of filtering and distance range on speed estimation accuracy will also be analyzed.

## Deliverables

- Final Project Report (IEEE format)

- Functional System Diagram showing sensor, processing, and estimation pipeline

- Tables and figures presenting speed estimation accuracy and detection results

- Conclusion summary discussing achieved objectives and limitations

- Source code for data acquisition and processing

- Experimental plots and evaluation results

- Complete measurement data

## References

[1] N. Thong-un and W. Wongsaroj, “A Novel Ultrasonic Method for Measuring the Position and Velocity of Moving Objects in 3D Space,” Advances in Technology Innovation, vol. 7, no. 2, pp. 77–91, 2022.

[2] L. Alonso, J. P. Riera, and M. A. González, “Ultrasonic Sensors in Urban Traffic Driving-Aid Systems,” Sensors, vol. 11, no. 1, pp. 661–676, 2011.

[3] A. L. Bowler et al., “Ultrasonic sensing and machine learning for industrial process monitoring,” Ultrasonics, vol. 120, 2022.